## What Is gRIBI?

gRIBI (pronounced "gribby") is a standardized gRPC-based API that lets external clients insert, modify, or delete IPv4/IPv6 forwarding entries (routes) in a network device’s RIB (Routing Information Base), which then may be programmed into the FIB (Forwarding Information Base).

## How gRIBI Works (Simplified)

- Client connects to router/switch using gRPC.

- Client sends route entries (IPv4/IPv6) with next-hop/group definitions.

- The network device accepts the programming, validates it, and updates its routing table.

- gRIBI supports acknowledgement, persistence, and electability of clients.

## Key Concepts

| Concept | Description |
| --- | --- |
| RIB | Routing Information Base – the full table of routes |
| FIB | Forwarding Information Base – hardware-programmed forwarding entries |
| gRIBI client | The external application/controller programming routes |
| gRIBI server | The router/switch accepting gRIBI messages |
| Election ID | Determines which client is active in multi-controller setups |
| ModifyRequest | Main RPC method to insert/delete route entries |

Basic gRIBI Programming Flow

Example: Add a Static Route

Assume you want to program a route to 10.0.0.0/24 via next hop 192.168.1.1.

You would:

- Define NextHop

- Create NextHopGroup referencing that NextHop

- Add IPv4Entry with prefix 10.0.0.0/24 and reference to the NextHopGroup

This is done via the Modify() RPC call in gRIBI.

let's walk through the gRIBI programming flow for a simple topology, showing how to define and link the components: NextHop (NH) → NextHopGroup (NHG) → IPv4 Prefix Entry.

Step-by-Step gRIBI Route Programming Flow

Example Topology:

We’ll program R1 to forward traffic to 10.10.10.0/24 via two next-hops: 192.168.0.2 (R2) and 192.168.0.3 (R3).

Building Blocks in gRIBI

| Component | Purpose |
| --- | --- |

| NextHop (NH) | Defines one next-hop IP (or interface) |
| --- | --- |

| NextHopGroup | A group of one or more NHs (load balancing) |
| --- | --- |

| IPv4Entry | The route (prefix) tied to an NHG |
| --- | --- |

Programming Flow

Step 1: Set Election ID (if multiple clients might control the RIB)

Step 2: Add NextHops

Step 3: Add NextHopGroup (NHG)

This creates a simple ECMP group over NH1 and NH2.

Step 4: Add IPv4 Prefix Entry

Now R1 will forward traffic to 10.10.10.0/24 via both NHs in the group.

Step 5: Optional: Confirm Programming

You should receive a ModifyResponse with status: OK and status_code: InstalledInRIB.

Summary of Flow

  Create NextHops → define where to send traffic.

  Group NHs into NHG → for ECMP/failover.

  Bind NHG to a prefix (route) → to direct actual traffic.

let's now enhance the earlier example by adding a backup path using gRIBI's backup_group feature. This allows one NHG to act as a fallback in case the primary NHG becomes unreachable (e.g., due to link failure or next-hop unreachability).

Topology with Primary and Backup Paths

Goal:

- Forward 10.10.10.0/24 via NHG 100 (containing NH1 and NH2).

- If NHG 100 fails, switch to NHG 200 (containing NH3).

Programming Flow with gRIBI

Step 1: Set Election ID

Step 2: Add NextHops

Step 3: Add Primary NHG (ID = 100)

Step 4: Add Backup NHG (ID = 200)

Step 5: Add IPv4Entry with Backup

This creates a resilient route:

- Primary path via NHG 100 (NH1 + NH2)

- Backup path via NHG 200 (NH3), activated only if NHG 100 fails

  Under normal conditions, traffic goes to R2 or R3 (NH1/NH2).

  If both NH1 and NH2 become unreachable, R1 automatically switches to NH3 (backup group).

Let's go deeper and show how to program routes in gRIBI using interface names and optionally MAC addresses — useful for Layer 2 / L3 hybrid setups or when directly specifying the outgoing interface instead of relying only on IP next-hops.

Topology (Using Interfaces & MACs)

Programming Flow Using Interface Names & MAC (via gRIBI)

You’ll use `NextHop` entries with either:

**IP-based** (normal)

**Interface-based** (Layer 2 or directly connected)

**MAC-based** (optional, if you're managing L2 manually)

Step 1: Add NextHop via Interface (No IP)

This creates a next-hop that forwards directly via interface et-1/0/0

Optional: Add MAC Address for NH (Layer 2 NH)

Some gRIBI implementations let you specify a MAC for more control:

Use this only if you need full control over L2 forwarding (e.g., static ARP-like behavior).

### You Provide:

- The egress interface name (e.g., eth0)

- The destination MAC address (e.g., the next-hop router's MAC)

This instructs the router to:

Forward the packet out of the specified local interface, using the given MAC address as the destination Ethernet header.

This is equivalent to a static ARP entry + static route:

- It bypasses ARP resolution.

- The router directly uses the given MAC on that interface.

Step 2: Add Backup NextHop via et-3/0/0

Step 3: Create NHGs

Primary NHG (ID 100)

Backup NHG (ID 200):

Step 4: Add Route with Backup

Final Behavior

  Traffic to 10.20.30.0/24 goes via eth1 to R2.

  If R2 is unreachable (e.g., eth1 down), traffic reroutes to eth3 to reach R3.

  If MAC is specified, gRIBI uses L2 forwarding (bypasses ARP resolution).

Let's walk through how to do IP-in-IP tunneling using gRIBI, which involves programming routes so that packets to a specific destination are encapsulated inside another IP packet — enabling use cases like VPNs, traffic engineering, or service chaining.

What Is IP-in-IP Tunneling?

P-in-IP tunneling encapsulates one IP packet inside another.
This creates a "tunnel" from one router to another across an intermediate network.

  Outer IP header → for transport across the tunnel.

  Inner IP header → original source and destination.

Topology Example: IP-in-IP Tunnel via gRIBI

Goal:

- On R1, create a route to 10.10.10.0/24 (Host B) that sends traffic via an IP-in-IP tunnel through R2.

- The outer tunnel destination is R2's loopback (192.168.100.1).

- Encapsulate traffic inside tunnel using gRIBI.

gRIBI Programming Components

To create this, you use gRIBI’s:

- Encapsulating NextHop: instructs the router to encapsulate traffic using IP-in-IP.

- NextHopGroup: groups one or more tunnel next-hops.

- IPv4Entry: points to NHG.

Step-by-Step Programming

- Define Tunnel NextHop with IP-in-IP Encapsulation

This tells R1 to:

- Encapsulate packets with an outer IP header from 192.168.100.2 to 192.168.100.1.

- Create NextHopGroup for the Tunnel

- Add Route for Final Destination (Inner IP)

This means:
"Send all traffic to 10.10.10.0/24 via tunnel to 192.168.100.1 (encapsulate it)."

## Router Behavior

- R1 receives traffic for 10.10.10.0/24.

- Matches it to NHG 100 → NH index 1.

- R1 encapsulates original packet in IP tunnel (src: 192.168.100.2, dst: 192.168.100.1).

- Sends it over the underlay network to R2.

- R2 decapsulates and forwards to final destination.

You're now touching on advanced gRIBI route programming, specifically

How to program a NextHop that lands in a different VRF after IP-in-IP decapsulation.

This is a real-world use case for:

- Service chaining

- Multi-tenant isolation

- Policy-based routing after tunnel termination

Let’s break it down.

TOPOLOGY: Decap and Forward to a Different VRF

Goal:

- Traffic for 10.10.10.0/24 comes in via IP-in-IP tunnel to 192.168.100.1 (R1's tunnel endpoint).

- After decapsulation, we want R1 to switch to VRF vrf-customer1 and forward based on RIB in that VRF.

## How It Works in gRIBI

You need to:

- Program the tunnel in the underlay VRF (e.g., vrf-underlay)

- After decap, forward the inner packet into a different VRF (e.g., vrf-customer1)

- Program NextHop in target VRF (i.e., what to do next)

## Step-by-Step: Programming Flow

### 🔹 Step 1: Program IP-in-IP Tunnel NextHop in Underlay VRF

This causes encapsulation and sends traffic to tunnel endpoint 192.168.100.1.

🔹 Step 2: Program NHG in Underlay

🔹 Step 3: Route in Underlay VRF

🔹 Step 4: Decap and Forward to VRF

At the decap router (R1, tunnel endpoint):

You configure post-decap next-hop into a different VRF like this:

This means: After stripping off the outer IP header (IP-in-IP), switch to vrf-customer1 for further lookup.

### 🔹 Step 5: Final NHG and IPv4Entry in Customer VRF

In vrf-customer1, define a route for the inner destination:

## Summary

| Step | Action |
| --- | --- |
| 1 | Encapsulate in underlay VRF |
| 2 | Decapsulate at tunnel endpoint |
| 3 | Redirect post-decap traffic to another VRF |
| 4 | Forward based on new VRF RIB |

An advanced and realistic service-chaining scenario using gRIBI.

We'll go through a full example that covers:

### Three critical steps:

- Encapsulate in a tunnel in VRF-A

- Decapsulate at a tunnel endpoint, then switch to VRF-B

- Post-decap: Forward to a next-hop (or encapsulate again) in VRF-C

Full Service Chain Topology (VRF-A → VRF-B → VRF-C)

## COMPONENTS OVERVIEW

| Action | VRF | What |
| --- | --- | --- |
| Encapsulate | vrf-a | Tunnel traffic to remote |
| Decapsulate | vrf-b | Strip tunnel and switch context |
| Forward | vrf-c | Final next-hop, or another tunnel |

## Step-by-Step gRIBI Configuration

### 🔹 1. Encapsulate in VRF-A

Traffic to 10.10.10.0/24 is encapsulated and sent toward 192.168.100.1 via VRF-A

### 2. Decapsulate in VRF-B and Switch to VRF-C

At the tunnel endpoint:

Decapsulated packet is handed off to VRF-C for further processing.

### 3. In VRF-C: Forward Normally or Re-Encapsulate

Example 1: Forward normally to destination

Example 2: Encapsulate again (re-tunnel)

## Recap of the Flow

| Stage | Action | VRF |
| --- | --- | --- |
| 1 | Encapsulate tunnel to 192.168.100.1 | vrf-a |
| 2 | Decapsulate IP-in-IP, switch VRF | vrf-b → vrf-c |
| 3 | Forward to next-hop or re-encapsulate | vrf-c |

let's dive into a more advanced, real-world gRIBI scenario that combines:

Multi-hop tunneling, service chaining, policy routing, and VRF transitions

This goes beyond simple encapsulation and includes NAT zones, firewall bypass, and layered service functions — all programmable via gRIBI.

Advanced Topology: Multi-hop, Multi-VRF Service Chain with Tunnels

### Goals:

- Traffic to `10.10.10.0/24` follows:

- Tunnel 1 (R1 → R2) in `ingress` VRF

- Decap, policy apply, Tunnel 2 (R2 → R3) in `services` VRF

- Decap and deliver to real host in `egress` VRF

- Full **VRF hopping**, **multiple encapsulations**, and **programmable service insertion**

Step-by-Step gRIBI Programming

Router A: IP-in-IP Tunnel 1 (Encap in `ingress` VRF)

Encapsulates traffic to 10.10.10.0/24 in Tunnel 1

### Router B: Decap Tunnel 1 → Switch to services VRF → Re-encap Tunnel 2

#### Step 2.1: Decap and switch to services

2.2: In services VRF: Re-encapsulate via Tunnel 2 to Router C

Service inspection complete; forward via Tunnel 2

Router C: Final Decap → Deliver to Final Host in egress VRF

3.1: Decap Tunnel 2 → switch to egress VRF

3.2: Deliver in egress VRF

Traffic finally reaches the real host at 10.10.10.1.

## End-to-End Flow Summary

| Step | VRF | Action |
| --- | --- | --- |
| 1 | ingress | Encap Tunnel 1 → R2 |
| 2 | ingress | Decap Tunnel 1 → switch to services |
| 3 | services | Encap Tunnel 2 → R3 |
| 4 | services | Decap Tunnel 2 → switch to egress |
| 5 | egress | Deliver to final IP |

## Why This Is Complex & Powerful

- Multiple tunnels, each in different VRFs

- Layered service chains with full decap/encap cycles

- Flexible network_instance switch after tunnel ops

- Can support policy-based routing, firewall NAT, or DPI chaining

You're really diving deep, and you're already hitting the top-tier use cases for programmable networks with gRIBI. After multi-hop tunneling, service chaining, and VRF transitions, we’re into territory that's at the cutting edge of SDN-driven architectures. Here's a set of even more advanced and specialized use cases you might explore:

## Beyond Multi-hop & VRF: Advanced gRIBI Use Cases

### 1. Segment Routing with Policy-Based NextHops

- Use gRIBI to define custom paths using segment identifiers (SIDs) or labels.

- Example: Force traffic through security, then telemetry, then exit.

- Requires MPLS/SRv6-capable infrastructure.

Use: mpls_label_stack in NextHop

### 2. Topology-Aware Load Balancing (ECMP + Affinity)

- Dynamically weight NextHops in an NHG based on real-time link health or traffic profiles.

- gRIBI allows adjusting weight fields on-the-fly.

Use for: Application-aware routing, real-time SLA optimization

### 3. Multicast Tree Steering via gRIBI

- Program IPv4MulticastEntry and NextHopGroup to control multicast delivery trees.

- Example: Send live video to multiple service zones via distinct VRFs or tunnels.

Use: multicast_group_id + per-interface next-hops

### 4. SFC (Service Function Chaining) with Reclassification

- Chain through DPI → Firewall → NAT → Analytics → Exit.

- After each decap or policy stage, reclassify and reprogram new gRIBI paths.

- Possible via network_instance handoff and conditional route injection.

Think: Programmable intent-based policy steering

### 5. Remote NH Resolution via Recursive Lookups

- NextHop’s IP is not directly reachable? gRIBI can chain lookups:

  - NH points to an IP

  - That IP is resolved via another IPv4Entry in another VRF or routing table

Example: Recursive tunnel → GRE → IPNH → Interface

### 6. Programmable Failover Trees

- Use backup_group + telemetry feedback to precompute N paths:

  - If NHG 1 fails → switch to NHG 2 → NHG 3, etc.

- Works beautifully for 5-nines traffic, disaster recovery, SD-WAN

Bonus: Combine with BFD session monitoring for tight-loop detection

### 7. Geo-Aware Policy Routing with gRIBI

- Inject routes to data centers, branches, or services based on geolocation or performance.

- Tie in to external policy engines (intent → gRIBI injection)

Use with: PCE controllers or gRPC-based policy agents

### 8. Zero-Trust Service Insertion + Telemetry Splitting

- Split mirrored traffic at decap into two paths:

  - One goes to analyzer

  - Other continues to destination

- All via parallel NHGs, gRIBI-programmed

## Want to Go Further?

You might also explore:

- Closed-loop automation (gRIBI + streaming telemetry + control feedback)

- Intent-to-path translation (via PCE controller or policy engine)

- End-to-end slicing using VRF + tunnel isolation

- Hybrid cloud interconnect automation using BGP + gRIBI fusion

Closed-loop automation with gRIBI is where programmable networking meets autonomous control — an ideal setup for:

- Self-healing networks

- Real-time optimization

- SLA enforcement

- Traffic steering based on telemetry

What is Closed-Loop Automation (CLA)?

Closed-loop automation means:

The network configures itself, monitors itself, and adjusts itself — continuously and automatically — based on real-time feedback.

## Core Components

| Component | Role |
| --- | --- |
| gRIBI | Programs the FIB (forwarding behavior) — e.g., routes, tunnels |
| Streaming Telemetry | Continuously exports state (e.g., interface drops, RTT, NH liveliness, etc.) |
| Analytics + Control Engine | Ingests telemetry, computes new intent or policies |
| Policy → gRIBI Feedback | Pushes back new next-hop routes, load balancing, or failovers |
| High-LevelArchitecture |  |

Example Scenario: ECMP Weights Based on Latency

### Goal:

- Route 10.10.10.0/24 via both Router2 and Router3

- Adjust ECMP weights dynamically based on RTT latency

## Step-by-Step Implementation

### 1️. Setup ECMP NHG via gRIBI

2️ Collect RTT via gNMI Telemetry

3️ Control Logic in Python (or any language)

4️ Push NHG Weight Update via gRIBI

## Advanced Feedback Examples

- Interface error spikes → Reroute away from bad links

- BFD session down → Trigger automatic failover (via backup_group)

- Packet drops > threshold → Inject temporary high-priority path

- Traffic type (e.g., video) → Route via low-jitter paths

## Integrate AI/ML

- Use historical telemetry + ML to predict congestion or failures

- Adjust NHGs before problems occur

- Combine with reinforcement learning to learn optimal paths over time

Let's build a closed-loop automation system using gRIBI, streaming telemetry, and Python. This system will dynamically adjust routing paths based on real-time latency measurements.

## Components Overview

- gRIBI Client: Programs the forwarding plane with new next-hop groups (NHGs).

- Telemetry Collector: Gathers real-time latency data from network devices.

- Control Logic Engine: Analyzes telemetry data and computes optimal routing decisions.

- gRIBI Feedback Loop: Sends updated routing information back to the network devices.

## Step 1: Install Dependencies

Ensure you have the necessary Python packages:

pip install grpcio pygnmi

## Step 2: Sample Telemetry Feed

We'll simulate latency data for two paths:

- Path A: Latency = 15ms

- Path B: Latency = 35ms

In a real-world scenario, this data would be collected using gNMI from network devices.

Step 3: Control Logic Engine

The control logic will analyze the telemetry data and adjust the NHG weights accordingly.

Step 4: gRIBI Client to Program NHGs

This function will send the updated NHG weights to the network devices using gRIBI.

------------ PYTHON CODE -----------

import grpc

from google.protobuf import any_pb2

from gribi_pb2 import *

def update_nhg_weights(nhg_weights):

# gRIBI server address

server_address = 'localhost:50051'

# Create a gRIBI client

channel = grpc.insecure_channel(server_address)

stub = GribiStub(channel)

# Create a NextHopGroup message

nhg = NextHopGroup(id=100)

for path, weight in nhg_weights.items():

nh = NextHop(index=1, ip_address=f"192.168.1.{path[-1]}", weight=weight)

nhg.next_hops.append(nh)

# Create an IPv4Entry message

ipv4_entry = IPv4Entry(prefix="10.10.10.0/24", next_hop_group=nhg.id)

# Create a Request message

request = Request(

operation=OperationType.ADD,

ipv4_entry=ipv4_entry,

next_hop_group=nhg

)

# Send the request to the gRIBI server

response = stub.Modify(request)

print(f"gRIBI Response: {response}")

-------------------------------------------------

Step 5: Closed-Loop Automation

Now, we'll integrate everything into a closed-loop system that continuously monitors latency and adjusts routing paths.

-------------- PYTHON CODE --------------

import time

def closed_loop_automation():

while True:

# Step 1: Collect telemetry data

telemetry_data = {

"path_a": {"latency": 15},

"path_b": {"latency": 35}

}

# Step 2: Analyze telemetry data

nhg_weights = adjust_nhg_weights(telemetry_data)

# Step 3: Program new NHG weights

update_nhg_weights(nhg_weights)

# Wait before the next iteration

time.sleep(30)

# Start the closed-loop automation

closed_loop_automation()

## Testing the System

To test this system:

- Run the gRIBI server on your network device.

- Execute the Python script on a machine that can reach the gRIBI server.

- Observe the routing changes on the network device as the script adjusts NHG weights based on simulated latency data.

## Next Steps

- Integrate Real Telemetry: Replace the simulated telemetry data with real-time gNMI data from your network devices.

- Enhance Control Logic: Implement more sophisticated algorithms for path selection, considering factors like bandwidth, jitter, and packet loss.

- Implement Failover Mechanisms: Add logic to handle link failures and reroute traffic accordingly.

Integrating AI/ML into closed-loop network automation with gRIBI is a powerful leap forward. You're essentially moving from reactive automation to predictive and proactive optimization.

## Objective

Use historical telemetry data to train ML models that can:

- Predict link congestion or failures

- Proactively adjust NHG (NextHopGroup) weights or paths via gRIBI

- Use Reinforcement Learning (RL) to optimize traffic paths over time

## Three AI/ML Strategies

| Strategy | Goal | Model Type |
| --- | --- | --- |
| 🧮 Supervised Learning | Predict link congestion/failure | Classification/Regression |
| 🧪 Time Series Forecasting | Predict latency or drop trends | LSTM, ARIMA |
| 🎮 Reinforcement Learning | Learn optimal pathing by experience | RL agents (Q-learning, PPO) |

Example: Predicting Congestion and Preemptively Adjusting NHGs

### 1️ Collect & Store Historical Telemetry

Use OpenConfig telemetry via gNMI to gather:

- Interface stats (in-octets, out-octets, in-errors, etc.)

- Queue drops or utilization

- Latency or RTT (via synthetic probes)

Store it in a time-series DB: Prometheus, InfluxDB, or even CSV/Parquet for training.

2️ Train a ML Model to Predict Congestion

Use sklearn or TensorFlow:

You now have a model that predicts whether an interface will be congested in the near future.

### 3️⃣ Use Model in Closed-Loop Automation

At runtime, use real-time telemetry to predict and act:

## Example: Reinforcement Learning for Path Optimization

Reinforcement Learning (RL) learns from interaction with the network environment:

### Agent Design:

- State = current traffic, latency, drops

- Action = choose NHG weights or paths

- Reward = minimize latency, avoid drops

Example using stable-baselines3:

At runtime:

obs = env.get_current_state()

action, _ = model.predict(obs)

env.apply_action(action)  # Adjust gRIBI NHGs

## Summary of Tools

| Function | Tools |
| --- | --- |
| Telemetry collection | gNMI + pygnmi, Prometheus, InfluxDB |
| Model training | sklearn, TensorFlow, PyTorch |
| Forecasting | statsmodels, Prophet, LSTM |
| RL | stable-baselines3, Ray RLlib |
| gRIBI control | grpcio + custom client with proto |

Let's build a full working Python notebook that integrates machine learning (ML) for predicting network congestionand utilizes gRIBI for real-time routing adjustments. This system will:

- Train a model using historical telemetry data.

- Predict congestion in real-time.

- Adjust routing proactively via gRIBI.

## Prerequisites

Ensure you have the following Python packages installed:

Bash

------

pip install grpcio pygnmi scikit-learn pandas

You'll also need access to a gRIBI server for programming the forwarding plane.

## Step 1: Collect and Prepare Telemetry Data

For this example, we'll simulate telemetry data. In a real-world scenario, you'd collect this data using gNMI from your network devices.

Python

-------

import pandas as pd

import numpy as np

# Simulate telemetry data

data = {

'timestamp': pd.date_range(start='2025-04-01', periods=100, freq='H'),

'out_octets': np.random.randint(100000, 1000000, 100),

'latency_ms': np.random.uniform(10, 50, 100),

'drops': np.random.randint(0, 10, 100),

'congested': np.random.choice([0, 1], 100)  # 0 = not congested, 1 = congested

}

df = pd.DataFrame(data)

df.set_index('timestamp', inplace=True)

df.head()

## Step 2: Train a Machine Learning Model

We'll use a Random Forest Classifier to predict congestion based on telemetry features.

Python

-------

from sklearn.ensemble import RandomForestClassifier

from sklearn.model_selection import train_test_split

from sklearn.metrics import classification_report

# Features and target

X = df[['out_octets', 'latency_ms', 'drops']]

y = df['congested']

# Split data

X_train, X_test, y_train, y_test = train_test_split(X, y, test_size=0.2, random_state=42)

# Train model

model = RandomForestClassifier(n_estimators=100, random_state=42)

model.fit(X_train, y_train)

# Evaluate model

y_pred = model.predict(X_test)

print(classification_report(y_test, y_pred))

## Step 3: Real-Time Prediction and Routing Adjustment

Now, we'll simulate real-time telemetry and adjust routing based on the model's predictions.

python

-------

import grpc

from google.protobuf import any_pb2

from gribi_pb2 import *

# Simulate real-time telemetry

real_time_telemetry = {

'out_octets': 500000,

'latency_ms': 30,

'drops': 2

}

# Prepare feature vector

features = np.array([[real_time_telemetry['out_octets'], real_time_telemetry['latency_ms'], real_time_telemetry['drops']]])

# Predict congestion

congestion_pred = model.predict(features)[0]

# Adjust routing if congestion is predicted

if congestion_pred == 1:

print("Congestion predicted: rerouting traffic")

# gRIBI client setup

server_address = 'localhost:50051'  # Replace with your gRIBI server address

channel = grpc.insecure_channel(server_address)

stub = GribiStub(channel)

# Create NextHopGroup message

nhg = NextHopGroup(id=100)

nhg.next_hops.append(NextHop(index=1, ip_address="192.168.1.2", weight=70))

nhg.next_hops.append(NextHop(index=2, ip_address="192.168.2.2", weight=30))

# Create IPv4Entry message

ipv4_entry = IPv4Entry(prefix="10.10.10.0/24", next_hop_group=nhg.id)

# Create Request message

request = Request(

operation=OperationType.ADD,

ipv4_entry=ipv4_entry,

next_hop_group=nhg

)

# Send request to gRIBI server

response = stub.Modify(request)

print(f"gRIBI Response: {response}")

else:

print("No congestion predicted: maintaining current routing")

## Step 4: Continuous Monitoring and Adjustment

For continuous operation, you can set up a loop that collects real-time telemetry, makes predictions, and adjusts routing accordingly.

python

--------

import time

while True:

# Simulate real-time telemetry

real_time_telemetry = {

'out_octets': np.random.randint(100000, 1000000),

'latency_ms': np.random.uniform(10, 50),

'drops': np.random.randint(0, 10)

}

# Prepare feature vector

features = np.array([[real_time_telemetry['out_octets'], real_time_telemetry['latency_ms'], real_time_telemetry['drops']]])

# Predict congestion

congestion_pred = model.predict(features)[0]

# Adjust routing if congestion is predicted

if congestion_pred == 1:

print("Congestion predicted: rerouting traffic")

# gRIBI client setup

server_address = 'localhost:50051'  # Replace with your gRIBI server address

channel = grpc.insecure_channel(server_address)

stub = GribiStub(channel)

# Create NextHopGroup message

nhg = NextHopGroup(id=100)

nhg.next_hops.append(NextHop(index=1, ip_address="192.168.1.2", weight=70))

nhg.next_hops.append(NextHop(index=2, ip_address="192.168.2.2", weight=30))

# Create IPv4Entry message

ipv4_entry = IPv4Entry(prefix="10.10.10.0/24", next_hop_group=nhg.id)

# Create Request message

request = Request(

operation=OperationType.ADD,

ipv4_entry=ipv4_entry,

next_hop_group=nhg

)

# Send request to gRIBI server

response = stub.Modify(request)

print(f"gRIBI Response: {response}")

else:

print("No congestion predicted: maintaining current routing")

# Wait before next iteration

time.sleep(60)  # Adjust the sleep time as needed

## Next Steps

- Integrate real telemetry: Replace the simulated telemetry with actual data from your network devices.

- Enhance the ML model: Incorporate additional features such as jitter, bandwidth utilization, and historical trends.

- Implement failover mechanisms: Add logic to handle link failures and reroute traffic accordingly.